ACCESS CONTROL MANAGEMENT

Software Suite Guide

Release  6.9

Jul,26,2010

CONTENTS

Preface	4

Purpose of this Document	4

Audience	4

Part 1  Setup and Remove applications	4

1.1  Setup applications	4

1.1.1  Setup Microsoft.Net	5

1.1.2  Setup Access Control software	7

1.2  Remove applications	9

Part 2  Login	11

Part 3 Adding and Editing Basic Configure	13

3.1  Add Department	13

3.2  Add and Edit a Personnel	14

3.3  Import customer's information from Excel	15

Part 4  Access Control	16

4.1  Configuration	16

4.1.1  Hardware Configure	16

4.1.2  Access Privilege	17

4.1.3  Rename Door	20

4.1.4  Set Open Delay	20

4.2   Basic Operate	21

4.2.1  Check controller’s Info	21

4.2.2  Upload Setting	22

4.2.3  Monitor	22

4.2.4  Download	24

4.2.5  Locate	25

4.2.6  Persons Inside	26

4.2.7  Query Records	26

Part 5  Tools	27

5.1  Lost Card register	27

5.2  Change Password	27

5.3  Operator Management	28

5.4  DB Compact/Repair	30

5.5  Interface Locked	30

5.6  Option	31

Part 6  Extended Function	32

6.1  Time profile	35

6.2  Peripheral control	38

6.3  PassWord Management	39

6.4  Anti-passback	41

6.5  InterLock	42

6.6  Activate Multi-card access	43

6.7  Timing auto-operation management	44

6.8  Timing collect data	45

6.9  Periodically Upload Privilege	46

6.10  Log Query	47

6.11  First Card Open	48

6.12  Monitoring Map	49

6.13  Camera Monitoring	50

6.14  Check Access Manger	54

6.15  Meeting Sign	57

Part 7  Module:Attendance	61

7.1  Normal shift configuration	61

7.2  Holiday	62

7.3  Leave/Business Trip	63

7.4  Manual Sign In	64

7.5  Report	65

Part 8  Module:Patrol	66

8.1  Patrol Configure	67

8.2  Patrolman	67

8.3  Patrol Point	68

8.4  Patrol Route	69

8.5  Patrol Task	70

8.6  Patrol Report	71

8.7  Patrol Statistics Report	71

Part 9  Const meal	72

9.1  Meal setup	72

9.1.1  Readers for Meal	72

9.1.2  Meal Rules	73

9.1.3  Meal Period	73

9.2  Meal Report	74

Part 10  Excursus:	75

10.1  RS232/RS485	75

10.2  TCP/IP	76

10.3  Complex network	76

# Preface

## Purpose of this Document

The Software Suite Guide provides the procedures and information necessary to use Version 6.9 of the Access Control System. These procedures guide you to use the following groups of functions:

Badging

Hardware configuration

Database configuration

Monitoring

Reporting

Administration

## Audience

This direction is written for two kind of readers .Part 1 is opened for actual operation Access Control System’s user,such as security personnel .Part 2 is opened for an administrator who first configures the system.

# Part 1  Setup and Remove applications

## 1.1  Setup applications

First puts in the CD to CD-ROM, then run the applications of setup () .

### 1.1.1  Setup Microsoft.Net

When in your computer has not installed Microsoft.NET Framework, The system can from next step for you to install Microsoft.NET Framework 1.1.

If your computer has installed Microsoft.NET Framework, The system will from next step for you to install Access Control System.

### 1.1.2  Setup Access Control software

After the installment completes, the desktop automatically will create、.

## 1.2  Remove applications

# Part 2  Login

Click the  or run All Programs << iCCard << Access Control & Attendance V6.9.

It will open up the Login windows .As follows:

By default, the user name required is “ abc ”, the password required is “123”.

It is recommend to change the name and password at the first use.

After login , It will show the main windows . as follows:

At you first login ,it will shows a basic guide. If you do not have the experience, you may complete the basic operation and the establishment under this guide's direction. But, we suggested that you’d better close the basic guide ,at the same time you should read the manual carefully to familiar with the operation of software.

After close the " basic guide", the operation windows as follows.

# Part 3 Adding and Editing Basic Configure

## 3.1  Add Department

Select  Basic Configure > Departments from the menu bar

click the  to create a new department.

Click the  to add a new section under a department.

## 3.2  Add and Edit a Personnel

Select  Basic Configure > Personnel from the menu bar

After you input all information about the user, click this button, the system had add a user .At the same time ,it will remain show the user windows and wait for you input the next user’s information .

## 3.3  Import customer's information from Excel

Select Personnel << Ctrl + Shift + Q << Import From Excel

If you want to use Excel importing customer’s information, please edit the document of userinfo.xls first., the userinfo.xls document is located on the control at gate manages software (C:\Program Files\iCCard),showing as this:

Remark: The card number must be more than three numbers, department and section can with"\" for box off a sign. For example: MIS\DEV

Import customer's information from Excel was successfully, information hints, click an assurance .

# Part 4  Access Control

## 4.1  Configuration

Your organization can prevent material or information robbery, by limiting / supervising the access to all or part of your facility (lab, computer room, or storage areas) to authorized persons, during specific time periods.

Smart multi-technology controllers, linked to advance identification systems, are programmed to control "who is going where and when". Each person is equipped with a personalised card or

another ID that controls access.

When a badge holder needs permission to access a particular area, the information is relayed

from the reader to the controller. The controller either grants or refuses access according to the

parameters defined (access authorization, time zones, etc.) The operations are then sent to the

PC and listed in the backlog and the journal.

Access control parameters are mainly defined in the “ Parameter” section of the application.

### 4.1.1  Hardware Configure

Select  Basic Configure << Controllers  from the menu bar

click the  to add the controllers into system.

The product S/N  (namely each controller serial number) may check on the PCB’s label S/N:*****  of  the controller . please fills that five numerals.

Note: If the product S/N you fills and the PCB's label S/N:***** are different ,it will cause the software can't communicate with the controllers.

If you want to see the setting details , please consult the Excursus .

### 4.1.2  Access Privilege

Select Access Control << Access Privilege  from the menu bar or shortcut.

After you click this button ,and then basic operate << upload ,the selected users can through selected doors .

After you click this button, the selected users can through selected doors .

After you click this button ,and then basic operate << upload ,the selected users can’t  through selected doors .

After you click this button , the selected users can’t  through selected doors .

: If user A and the user B have the same privileges, we can copy user A’s privileges to user B.

After you add the all privilege into the system ,you must go to the basic operate << upload to Upload Setting(Part 4.2.2).

### 4.1.3  Rename Door

Select Basic Operate << Console ,

Select the door first and then Right Click pop-up menu.

### 4.1.4  Set Open Delay

Select Basic Operate << Console

Select the door first and then Right Click pop-up menu.

## 4.2   Basic Operate

Select Basic Operate << Console from the menu bar .The console windows contain many basic operate for controller .For example ,check info  Adjust Time  Upload  Monitor  Collect Data etc.

### 4.2.1  Check controller’s Info

Select Basic Operate << Console from the menu bar or the shortcut of Basic Operate << Check

It will show the control’s basic information .such as  the amount of records and privileges ,door state ,control state ,open delay(sec) .If the controller is good Communicate with computer , the door label’s color is green , otherwise the door label’s color is red .

### 4.2.2  Upload Setting

Select  Basic Operate << Console from the menu bar or shortcut of Basic Operate << Upload

After you set the access controller’s information , you must upload the database’ configuration to access controllers.

### 4.2.3  Monitor

Select  Basic Operate << Console from the menu bar or shortcut of Basic Operate << Monitor

Attention : how to display user’s photo at Monitor

First: You must rename the user’s photo name with his or her Card Number.

For example: the photo name is 8862127.jpg. In actual 8862127 is Card Number.

Second: When you adding user, the user’s Card Id is the photo name just what you renamed .For example: 8862127 is the photo’s name.

At the monitor window

When the user swiping card, the monitor window could show the user’s photo at the same time.

### 4.2.4  Download

Select Basic Operate << Console from the menu bar or shortcut of Basic Operate << Download

: Gather the access controller’s records to database.

=  +

### 4.2.5  Locate

At the Realtime download window, Select the door first and then Right Click pop-up menu, then choose Locate .you can carry out Personnel localization.

You can input the user’s Name or Card ID or Worker No into the User textbox ,it will indicate this person the latest record that what’s time which door the person entered. So you can know the place of this person at present.

### 4.2.6  Persons Inside

At the Realtime download, Select the door first and then Right Click pop-up menu .,then choose Persons Inside ,it will show the window of  Persons Inside the door.

### 4.2.7  Query Records

Select  Basic Operate << Query Card Records from the menu bar or the shortcut of  Basic Operate << Query Card Records

# Part 5  Tools

## 5.1  Lost Card register

When someone has lost him or her card, you must carry on the loss registration to the old card , and then redistributes a new card to someone .The steps as follows:

Select Basic Configure<<Lost Card register

## 5.2  Change Password

Change operator’s password.

Select Tools<< Change Password

## 5.3  Operator Management

Select Tools<< operator Management

: assign the executable operation and function to operators

: assign the Department that the operator can manage.

: assign the Door that the operator can manage.

## 5.4  DB Compact/Repair

Select Tools<< DB Compact/Repair

## 5.5  Interface Locked

The operator is afraid of someone will operate the software after she or he left ,he can interface locked the window ,the application run as also.

Select Tools<< Interface Locked

After he or she comes back , he or she inputs the password all right.

## 5.6  Option

Tools<< Option

Select Child Function (Access Control, Patrol, Other Shift Const Meal)

Select Environment(Office  ,House)

Select language  (English,Chinese)

Set Auto Login 
Set Director of Backup

# Part 6  Extended Function

Select  【tools】<<【Extended Functions】 from the menu bar

If you want to Activate the Extended Function . please input the password .

Please ask provider for password.

The extended function as follows:

## 6.1  Time profile

【Extended Functions】<<【Normal】<<【Active Time Profile】

Select Access Control << Time Profile

Select Basic Configure<<Access Control << Access Privilege << Change Privilege

## 6.2  Peripheral control

【Extended Functions】<<【Normal】<<【Peripheral control】

Select Basic Configure << Access Control  <<  Peripheral control

## 6.3  PassWord Management

【Extended Functions】<<【Normal】<<【Access Keypad】

Select Basic Configure<<Access Control<<Password Management

【Extended Functions】<<【Normal】<<【Supervisor PIN】

【Extended Functions】<<【Others】<<【Supervisor PIN】

## 6.4  Anti-passback

【Extended Functions】<<【Advanced】<<【Anti-pass back】

## 6.5  InterLock

【Extended Functions】<<【Advanced】<<【InterLock】

## 6.6  Activate Multi-card access

【Extended Functions】<<【Advanced】<<【Activate Multi-card access】

The first group has two peoples the second group has two people , the third group has four people . each group comes two people to be allowed to open the door.

## 6.7  Timing auto-operation management

【Extended Functions】<<【Advanced】<<【Timing auto-operation management】

## 6.8  Timing collect data

【Extended Functions】<<【Others】<<【Timing Download】

## 6.9  Periodically Upload Privilege

【Extended Functions】<<【Others】<<【Periodically Upload Privilege】

## 6.10  Log Query

【Extended Functions】<<【Event】<<【Log Query】

## 6.11  First Card Open

【Extended Functions】<<【Advanced】<<【First Card Open】

## 6.12  Monitoring Map

【Extended Functions】<<【Interface】<<【Monitoring Map】

## 6.13  Camera Monitoring

【Extended Functions】<<【Interface】<<【Camera Monitoring】

Select 【Tools】<<【Camera Management】

Add camera, click “New”, as shown:

Set the Readers corresponding Web Camera

Display Photo Compare Window. Please【Console】<<【Camera】

If you want to delete snap pictures and video files, please 【Tools】<<【Camera Management】<<【Delete Bmp-Video】.

If you want to set up video server's IP address, please 【Tools】<<【Camera Management】<<【Set Video IP】,as shown:

Please input "New IP Address" and re-input “Confirmed New IP”. Information for the show:

Set Video IP Successfully.

If the Information Display:

Set Video IP Failed.

Query Camera View, Select 【Query Card Records】<<【Camera View】

## 6.14  Check Access Manger

【Extended Functions】<<【Advanced】<<【Active Check Access Inside】

Select Tools << Check Access Manager

Set the department for opening doors by swpiping.

Set the deparments for outside swiping.

click “Edit”.

In the console “realtime-download”, external swipings will active pop-up message box, as followed:

## 6.15  Meeting Sign

【Extended Functions】<<【Interface】<<【Active Meeting Sign】

Select Meeting << Meeting Sign

Add the meeting, please click ,

Click   , modify the meeting.

Click   to delete the meeting.

click "Add Addr", as shown:

click "Add" ,

Click “Stat.”:

Click “Real-time Sign”:

# Part 7  Module:Attendance

The Access Control System had Activated the Attendance by default. If you want cancel this function ,please select 【Extended Functions】<<【Interface】<<【Don’t Display Attendance】

## 7.1  Normal shift configuration

## 7.2  Holiday

## 7.3  Leave/Business Trip

## 7.4  Manual Sign In

## 7.5  Report

Attendance Detail

Attendance Statistics Report

# Part 8  Module:Patrol

【Extended Functions】<<【Interface】<<【Display Patrol】

## 8.1  Patrol Configure

## 8.2  Patrolman

## 8.3  Patrol Point

## 8.4  Patrol Route

## 8.5  Patrol Task

## 8.6  Patrol Report

## 8.7  Patrol Statistics Report

# Part 9  Const meal

【Extended Functions】<<【Interface】<<【Const meal】

## 9.1  Meal setup

### 9.1.1  Readers for Meal

Select Readers for Meal from Optional Readers

### 9.1.2  Meal Rules

### 9.1.3  Meal Period

## 9.2  Meal Report

Meal Details Report

Meal Stat. report of Readers for Meal

Meal Stat. report of Users

# Part 10  Excursus:

## 10.1  RS232/RS485

## 10.2  TCP/IP

all controllers are in the same network..

## 10.3  Complex network

The controllers are separated into different network . Each controller must be assigned a unique IP address .